Media Contact:

Vladislav Alexiev

Manager Product Communications

Tel: +49 69 850928 354  E-mail: valexiev@kia-europe.com

2017 Kia Rio
Technical specifications (Europe)

Body and chassis

Five-door, five-seater hatchback, with all-steel unitary construction bodyshell. Choice of diesel and gasoline three- and four-cylinder engines, driving the front wheels via five- or six-speed manual or four-speed automatic transmission – depending on model

Engines

Gasoline

1.0-litre / 100 ps T-GDI gasoline (low)

Type			Three-cylinder in-line, turbocharged

Capacity		1.0-litres, 998 cc

Compression ratio	10.0:1

Bore and stroke		71.0 x 84.0 mm

Max power		100 ps (74 kW) @ 4,500 rpm

Max torque		172 Nm (127 lb ft) @ 1,500-4,000 rpm

Valves			12 (four per cylinder)

Fuel system		Direct injection

Emissions class		Euro Stage 6b

1.0-litre / 120 ps T-GDI gasoline (high)

Type			Three-cylinder in-line, turbocharged

Capacity		1.0-litres, 998 cc

Compression ratio	10.0:1

Bore and stroke		71.0 x 84.0 mm

Max power		120 ps (88 kW) @ 6,000 rpm

Max torque		172 Nm (127 lb ft) @ 1,500-4,000 rpm

Valves			12 (four per cylinder)

Fuel system		Direct injection

Emissions class		Euro Stage 6b

1.25-litre / 84 ps MPI gasoline

Type			Four-cylinder in-line, naturally-aspirated

Capacity		1.25-litres, 1,248 cc

Compression ratio	10.5:1

Bore and stroke		71.0 x 78.8 mm

Max power		84 ps (62 kW) @ 6,000 rpm

Max torque		122 Nm (90 lb ft) @ 4,000 rpm

Valves			16 (four per cylinder)

Fuel system		Multi-point injection

Emissions class		Euro Stage 6b

1.4-litre / 100 ps MPI gasoline

Type			Four-cylinder in-line, naturally-aspirated

Capacity		1.4-litres, 1,368 cc

Compression ratio	10.5:1

Bore and stroke		72.0 x 84.0 mm

Max power		100 ps (74 kW) @ 6,000 rpm

Max torque		133 Nm (98 lb ft) @ 4,000 rpm

Valves			16 (four per cylinder)

Fuel system		Multi-point injection

Emissions class		Euro Stage 6b

Diesel

1.4-litre / 77 ps CRDi diesel (low)

Type			Four-cylinder in-line, turbocharged

Capacity		1.4-litres, 1,396 cc

Compression ratio	16.0:1

Bore and stroke		75.0 x 79.0 mm

Max power		77 ps (57 kW) @ 4,000rpm

Max torque		240 Nm (177 lb ft) @ 1,500-2,000 rpm

Valves			16 (four per cylinder)

Fuel system		Common-rail direct injection

Emissions class		Euro Stage 6b

1.4-litre / 90 ps CRDi diesel (high)

Type			Four-cylinder in-line, turbocharged

Capacity		1.4-litres, 1,396 cc

Compression ratio	16.0:1

Bore and stroke		75.0 x 79.0 mm

Max power		90 ps (66 kW) @ 4,000 rpm

Max torque		240 Nm (177 lb ft) @ 1,500-2,500 rpm

Valves			16 (four per cylinder)

Fuel system		Common-rail direct injection

Emissions class		Euro Stage 6b

Transmissions

Five-speed manual transmission

Six-speed manual transmission

Four-speed automatic transmission

Gasoline

1.0 T-GDI (lo)	1.0 T-GDI (hi)	1.25 MPI	1.4 MPI

Manual			5sp		6sp		5sp		6sp

Automatic		---		---		---		4sp

Diesel

1.4 CRDi (lo)	1.4 CRDi (hi)

Manual			6sp		6sp

Gear ratios

Gasoline

1.0 T-GDI (lo)	1.0 T-GDI (hi)	1.25 MPI	1.4 MPI

MT		MT		MT		MT / AT

1			3.615		3.615		3.545		3.769 / 2.919

2			1.955		1.955		1.895		2.045 / 1.551

3			1.207		1.286		1.192		1.370 / 1.000

4			0.893		0.971		0.906		1.036 / 0.713

5			0.703		0.794		0.719		0.839 / ---

6			---		0.667		---		0.703 / ---

Reverse		3.545		3.700		3.636		3.700 / 2.480

Final drive		3.722*		4.059		4.600     	4.267 / 4.375

Diesel

1.4 CRDi (lo)	1.4 CRDi (hi)

MT		MT

1			3.769*		3.636

2			2.040*		1.962

3			1.257		1.257

4			0.905		0.905

5			0.702		0.702

6			0.596		0.596

Reverse		3.583		3.583

Final drive		3.250*		3.706

*with Eco pack

Drivetrains

Front-wheel drive (all models)

Suspension and damping

Front	Fully-independent by subframe-mounted MacPherson struts, coil springs and gas-filled shock absorbers, with anti-roll stabiliser bar

Rear	Semi-independent by CTBA (coupled torsion beam axle), coil springs and gas-filled shock absorbers, with anti-roll stabiliser bar

Steering

Type	Electric motor-driven rack-and-pinion power steering

Steering ratio	14.1:1

Turns (lock-to-lock)	2.63

Turning circle (m)	10.2

Brakes

1.0 T-GDI	1.25 MPI	1.4 MPI		1.4 CRDi

Size, front / rear (mm)	280 / 203	256 / 203	256 / 203	280 / 203

Type, front	Ventilated disc

Type, rear – standard	Drum

Type, rear – optional	Solid disc (262 mm)

Wheels and tyres

Standard	Steel 15-inch, 185/65 R15 tyres

Optional 		Alloy 15-inch, 185/65 R15 tyres

Alloy 16-inch, 195/55 R16 tyres

Alloy 17-inch, 205/45 R17 tyres

Spare			Tyre mobility kit or optional temporary spare wheel

Dimensions (mm)

Exterior

Overall length		4,065		Overall width*		1,725

Overall height		1,450		Wheelbase		2,580

Front overhang		830		Rear overhang		655

Front track		1,518**		Rear track    		1,524

Ground clearance	135 / 140***

*excluding door mirrors

**on 15-inch wheels

***with / without Low Fuel Consumption Pack (see ‘Fuel efficiency and emissions’ below)

Interior

1st row		2nd row

Head room		1,020		964

Leg room		1,070		850

Shoulder room		1,375		1,355

Hip room		1,344		1,330

Capacities

Fuel tank		45 litres

Luggage (VDA)		325 litres

980 litres (with rear seats folded down)

Weights (kg)

Gasoline

1.0 T-GDI (lo)	1.0 T-GDI (hi)	1.25 MPI	1.4 MPI (MT / AT)

Curb weight		1,155		1,160		1,110		1,130 / 1,158

Gross weight		1,600		1,620		1,560		1,580 / 1,600

Tow load, braked	1,000		1,110		910		1,000 / 800

Tow load, unbraked	450		450		450		450 / 450

Diesel

1.4 CRDi (lo)	1.4 CRDi (hi)

Curb weight		1,230		1,235

Gross weight		1,680		1,680

Tow load, braked	1,110		1,110

Tow load, unbraked	450		450

Performance

Gasoline

1.0 T-GDI (lo)	1.0 T-GDI (hi)	1.25 MPI	1.4 MPI (MT / AT)

Top speed		188		190		170		176 / 166

Acceleration, 0-100 kph	10.7		10.2		12.9		12.2 / 13.9

Diesel

1.4 CRDi (lo)	1.4 CRDi (hi)

Top speed		165		175

Acceleration, 0-100 kph	14.0		12.0

Fuel efficiency and emissions

Gasoline

1.0 T-GDI (lo)	1.0 T-GDI (hi)	1.25 MPI	1.4 MPI (MT / AT)

Fuel economy (combined, L/100km)

- 15-inch, LFC Pack	4.1		---		---		--- / ---

- 15-inch, Eco Pack	4.3		---		---		--- / ---

- 15-inch, LRR tyres*	---		4.7		4.7		--- / 5.9

- 15-inch		4.5		4.7		4.8		5.0 / 6.1

- 16-inch		4.5		4.7		4.8		5.0 / 6.1

- 17-inch		4.5		4.7		4.8		5.0 / 6.1

CO2 (g/km)

- 15-inch, LFC Pack	94		---		---		--- / ---

- 15-inch, Eco Pack	99		---		---		--- / ---

- 15-inch, LRR tyres*	---		107		106		--- / 135

- 15-inch		102		107		109		114 / 140

- 16-inch		102		107		109		114 / 140

- 17-inch		102		107		109		114 / 140

Diesel

1.4 CRDi (lo)	1.4 CRDi (hi)

Fuel economy (combined, L/100km)

- 15-inch, LFC Pack	3.4		---

- 15-inch, Eco Pack	3.5		---

- 15-inch, LRR tyres*	---		3.7

- 15-inch		---		3.8

- 16-inch		---		3.8

- 17-inch		---		3.8

CO2 (g/km)

- 15-inch, LFC Pack	88		---

- 15-inch, Eco Pack	92		---

- 15-inch, LRR tyres*	---		96

- 15-inch				98

- 16-inch		---		98

- 17-inch		---		98

*Michelin low rolling resistance tyres only; no LFC or Eco Packs

JANUARY 2017